Catalogue

Interface Functions	3

Callback Function Definition	3

Establish Connection with Device	4

Disconnect From Device	5

Get Informations of the Reader Module	6

Single Polling Read	7

Set “Select” Parameters	8

Set “Select” Mode	9

Read RFID Data Storage	10

Write RFID Data Storage	11

Lock RFID Data Storage	12

Kill RFID	13

Get Query Parameters	14

Set Query Parameters	15

Set Working Region	16

Set Working Channel	17

Get Working Channel	18

Set FHSS	19

Set Transmitting Power	20

Get Transmitting Power	21

Set Continuous Carrier Wave	22

Set Modem Parameters	23

Get Modem Parameters	24

Scan Jammer on RF Input	25

Scan RSSI	26

NXP ReadProtect/Reset ReadProtect	27

NXP Change EAS	28

NXP EAS_Alarm	29

NXP ChangeConfig	30

Impinj Monza QT	31

NetCfg_Open	32

NetCfg_Close	33

NetCfg_SearchForDevices	34

NET_DEVICE_CONFIG Structure	35

DEVICEHW_CONFIG Structure	36

DEVICEPORT_CONFIG Structure	37

NetCfg_GetInfo	38

NetCfg_SetInfo	39

NetCfg_FactoryReset	40

Interface Functions

Callback Function Definition

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD(WINAPI *Async_Receive)(BYTE Type, BYTE Command, DWORD ParamSize, LPBYTE ParamData); |  |  |  |
| Remarks |  |  |  |
| When there is notice created by devices, call this function to inform the host. |  |  |  |
| Parameter | DataType |  | Description |
| Type | Byte | out | Type 0x02；Notice frame: from device to host |
| Command | Byte | out | Type of command |
| ParamSize | DWORD | out | Length of content |
| ParamData | Byte[ ] | out | Content |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

Establish Connection with Device

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD Connect(BYTE ConnType, LPSTR ConnChar, Async_Receive Ar); |  |  |  |
| Parameter | DataType |  | Description |
| ConnType | Byte | in | Type of connection； 1;Serial ports； 2:USB； 3:TCP； |
| ConnChar | String | in | Content of connection； Serial ports:(“COM1”)； USB:(NULL)； TCP:(“IP Address:Port”) |
| Ar | Async_Receive | in | Callback function handle, in order to pass notification, pass NULL if notice is not needed. |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

Disconnect From Device

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD Disconnect(); |  |  |  |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

Get Informations of the Reader Module

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD GetModuleInfo(LPBYTE InfoType, LPSTR InfoData, LPDWORD DataSize) |  |  |  |
| Remarks |  |  |  |
| Get Informations of the reader module, such as hardware version, software version and manufacturer info. |  |  |  |
| Parameters | DataType |  | Description |
| InfoType | Byte | in,out | Type； Hardware version：0x00； Software version：0x01 |
| InfoData | String | out | Content |
| DataSize | DWORD | out | Length of content |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

Single Polling Read

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD ReadSingle(); |  |  |  |
| Remarks |  |  |  |
| If this function is called with a card attatched to the device, result will be passed by function Async_Receive. |  |  |  |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

Set “Select” Parameters

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD SetSelectParam(BYTE Target, BYTE Action, BYTE MemBank, DWORD Pointer, BYTE Truncated, LPBYTE MaskData, BYTE MaskSize); |  |  |  |
| Remarks |  |  |  |
| Set Select parameters and set Select mode to 0x02 at the same time. Select command should be sent before polling read the RFIDs. When multiple RFID exists, Select parameters are used to operate on single specific RFID. |  |  |  |
| Parameters | DataType |  | Description |
| Target | Byte | in |  |
| Action | Byte | in |  |
| MemBank | Byte | in | 00: RFU data storage area 01: EPC data storage area 02: TID data storage area 03: User data storage area |
| Truncated | Byte | in | 0x00(0x00 for Disable truncation，0x80 for Enable truncation) |
| MaskData | Byte[ ] |  | EPC Code |
| MaskSize | Byte |  | Length of MaskData |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

Set “Select” Mode

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD SetSelectMode(BYTE Mode); |  |  |  |
| Remarks |  |  |  |
| After setting Select parameters, call this function to set Select mode. |  |  |  |
| Parameters | DataType |  | Description |
| Mode | Byte | in | 0x00: do Select to specify single RFID before ALL operations. 0x01: no Select done before ANY operations. 0x02: do Select before all operations EXCEPT POLLING INVENTORY. |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

Read RFID Data Storage

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD ReadData(LPBYTE AccessPassword, BYTE MemBank, DWORD StartIndex, DWORD Length, LPBYTE PC, LPBYTE EPC, LPBYTE Data, LPDWORD Size); |  |  |  |
| Remarks |  |  |  |
| This function is used to read data with specific beginning address and length from memory bank of a single RFID. Unit size of the address offset SA and data length DL is the size of a WORD, which is also the length of 2 bytes or 16 bits. Select command must be sent before reading to choose RFID to be read from. If AccessPassword is made of zeros then Access command will not be sent. |  |  |  |
| Parameters | DataType |  | Description |
| AccessPassword | Byte[ ] | in | Access Password |
| MemBank | Byte | in | 00: RFU data storage area 01: EPC data storage area 02: TID data storage area 03: User data storage area |
| StartIndex | DWORD | in | Beginning address of data to be read |
| Length | DWORD | in | Length of data to be read |
| PC | Byte[ ] | out | PC of the RFID to be read |
| EPC | Byte[ ] | out | EPC of the RFID to be read |
| Data | Byte[ ] | out | Return data |
| Size | Byte[ ] | out | Length of return data |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

Write RFID Data Storage

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD WriteData(LPBYTE AccessPassword, BYTE MemBank, DWORD StartIndex, LPBYTE Data, DWORD Size, LPBYTE PC, LPBYTE EPC); |  |  |  |
| Remarks |  |  |  |
| This function is used to write data with specific beginning address and length to memory bank of a single RFID. Unit size of the address offset SA and data length DL is the size of a WORD, which is also the length of 2 bytes or 16 bits. Select command must be sent before writing to choose RFID to be write to. If AccessPassword is made of zeros then Access command will not be sent. |  |  |  |
| Parameters | DataType |  | Description |
| AccessPassword | Byte[ ] | in | Access Password |
| MemBank | Byte | in | 00: RFU data storage area 01: EPC data storage area 02: TID data storage area 03: User data storage area |
| StartIndex | DWORD | in | Beginning address of data to be written |
| Length | DWORD | in | Length of data to be written |
| PC | Byte[ ] | out | PC of the RFID to be written |
| EPC | Byte[ ] | out | EPC of the RFID to be written |
| Data | Byte[ ] | out | Return data |
| Size | Byte[ ] | out | Length of return data |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

Lock RFID Data Storage

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD LockUnlock(LPBYTE AccessPassword, LPBYTE LD, LPBYTE PC, LPBYTE EPC); |  |  |  |
| Remarks |  |  |  |
| Lock or Unlock Data Storage of a single RFID. Select command must be sent before locking or unlocking to choose RFID to be operated. |  |  |  |
| Parameters | DataType |  | Description |
| AccessPassword | Byte[ ] | in | Access Password |
| LD | Byte[ ] | in | The highest 4 bits of LD is preserved, and the remaining 20 bits contains the Payload of the Lock command, including Mask and Action. Mask and Action each takes 10 bits from higher bits to lower ones. For further information, please refer to section 6.3.2.11.3.5 of EPC Gen2 Protocol (v1.2.0). Mask stands for a mask code，Actions are only effective when corresponding mask bit is 1. There are 2 bits of Action in each data storage area，00~11，each means Open, Open permanently, Locked and Locked permanently. |
| PC | Byte[ ] | out | PC of the RFID to be operated |
| EPC | Byte[ ] | out | EPC of the RFID to be operated |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

Kill RFID

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD Kill(LPBYTE AccessPassword, LPBYTE PC, LPBYTE EPC); |  |  |  |
| Remarks |  |  |  |
| Kill a single RFID. Select command must be sent before this operation to specify single RFID. |  |  |  |
| Parameters | DataType |  | Description |
| AccessPassword | Byte[ ] | in | Access Password |
| PC | Byte[ ] | out | PC of the RFID to be operated |
| EPC | Byte[ ] | out | EPC of the RFID to be operated |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

Get Query Parameters

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD GetQuery(LPBYTE DR, LPBYTE M, LPBYTE TRext, LPBYTE Sel, LPBYTE Session, LPBYTE Target, LPBYTE Q); |  |  |  |
| Remarks |  |  |  |
| Get parameters related to Query command from firmware. |  |  |  |
| Parameters | DataType |  | Description |
| DR | Byte | out | =0：Mode8 =1：Mode64/3 Only Mode8 supported |
| M | Byte | out | =0：Mode1 =1：Mode2 =2：Mode4 =3：Mode8 Only Mode1 supported |
| TRext | Byte | out | =0：No pilot tone =1：Use pilot tone Only Use pilot tone supported |
| Sel | Byte | out | =0：ALL =1：ALL =2：~SL =3：SL |
| Session | Byte | out | =0：S0 =1：S1 =2：S2 =3：S3 |
| Target | Byte | out | =0：A =1：B |
| Q | Byte | out | 0-15； |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

Set Query Parameters

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD SetQuery(BYTE DR, BYTE M, BYTE TRext, BYTE Sel, BYTE Session, BYTE Target, BYTE Q); |  |  |  |
| Remarks |  |  |  |
| Set parameters related to Query command. |  |  |  |
| Parameters | DataType |  | Description |
| DR | Byte | in | =0：Mode8 =1：Mode64/3 Only Mode8 supported |
| M | Byte | in | =0：Mode1 =1：Mode2 =2：Mode4 =3：Mode8 Only Mode1 supported |
| TRext | Byte | in | =0：No pilot tone =1：Use pilot tone Only Use pilot tone supported |
| Sel | Byte | in | =0：ALL =1：ALL =2：~SL =3：SL |
| Session | Byte | in | =0：S0 =1：S1 =2：S2 =3：S3 |
| Target | Byte | in | =0：A =1：B |
| Q | Byte | in | 0-15； |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

Set Working Region

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD SetRegion(BYTE Region); |  |  |  |
| Remarks |  |  |  |
| Set work region of the reader. |  |  |  |
| Parameters | DataType |  | Description |
| Region | Byte | in | Region code 1：China 900MHz 2：USA 3：Europe 4：China 800MHz 6：South Korea |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

Set Working Channel

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD SetRfChannel(BYTE CH_Index) ; |  |  |  |
| Remarks |  |  |  |
| Set working channel of the reader. |  |  |  |
| Parameters | DataType |  | Description |
| CH_Index | Byte | in | Calculation formula，Freq_CH is the frequency of the channel： China 900MHz CH_Index = (Freq_CH-920.125M)/0.25M China 800MHz CH_Index = (Freq_CH-840.125M)/0.25M USA CH_Index = (Freq_CH-902.25M)/0.5M Europe CH_Index = (Freq_CH-865.1M)/0.2M South Korea CH_Index = (Freq_CH-917.1M)/0.2M |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

Get Working Channel

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD GetRfChannel(LPBYTE CH_Index); |  |  |  |
| Remarks |  |  |  |
| Get working channel of the reader. |  |  |  |
| Parameters | DataType |  | Description |
| CH_Index | Byte | in | Calculation formula，Freq_CH is the frequency of the channel： China 900MHz Freq_CH = CH_Index * 0.25M + 920.125M China 800MHz Freq_CH = CH_Index * 0.25M + 840.125M USA Freq_CH = CH_Index * 0.5M + 902.25M Europe Freq_CH = CH_Index * 0.2M + 865.1M South Korea Freq_CH = CH_Index * 0.2M + 917.1M |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

Set FHSS

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD SetFhss(BOOL Param); |  |  |  |
| Remarks |  |  |  |
| Set to or cancel frequency-hopping spread spectrum (FHSS). |  |  |  |
| Parameters | DataType |  | Description |
| Param | Bool | in | TRUE ：set FHSS FALSE ：cancel FHSS |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

Set Transmitting Power

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD SetPower(DWORD Power); |  |  |  |
| Remarks |  |  |  |
| Set transmitting power for current reader. |  |  |  |
| Parameters | DataType |  | Description |
| Power | DWORD | in | 2000，as 20dBm |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

Get Transmitting Power

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD GetPower(LPDWORD Power); |  |  |  |
| Remarks |  |  |  |
| Get transmitting power of current reader. |  |  |  |
| Parameters | DataType |  | Description |
| Power | DWORD | out | 2000，as 20dBm |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

Set Continuous Carrier Wave

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD SetCW(BOOL Param); |  |  |  |
| Remarks |  |  |  |
| Turn on or off transmitting continuous carrier wave. |  |  |  |
| Parameters | DataType |  | Description |
| Param | Bool | in | TRUE ：turn on FALSE ：turn off |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

Set Modem Parameters

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD SetModemPara(BYTE Mixer_G, BYTE IF_G, DWORD Thrd); |  |  |  |
| Remarks |  |  |  |
| Set modem parameters for current reader, including Mixer Gain, IF AMP Gain and Demodulation threshhold. |  |  |  |
| Parameters | DataType |  | Description |
| Mixer_G | Byte | in | 0x00：0(dB) 0x01：3(dB) 0x02：6(dB) 0x03：9(dB) 0x04：12(dB) 0x05：15(dB) 0x06：16(dB) |
| IF_G | Byte | in | 0x00：12(dB) 0x01：18(dB) 0x02：21(dB) 0x03：24(dB) 0x04：27(dB) 0x05：30(dB) 0x06：36(dB) 0x07：40(dB) |
| Thrd | DWORD | in | When the threshhold is lower, the minimum RSSI that could be demodulated is lower, yet more unstable. Demodulation will totally fail if RSSI is too low. On the contrary, higher threshhold means only higher RSSI could be demodulated and more stable. 432 is the lowest suggested value. |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

Get Modem Parameters

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD GetModemPara(LPBYTE Mixer_G, LPBYTE IF_G, LPDWORD Thrd); |  |  |  |
| Remarks |  |  |  |
| Get modem parameters of current reader, including Mixer Gain, IF AMP Gain and Demodulation threshhold. |  |  |  |
| Parameters | DataType |  | Description |
| Mixer_G | Byte | out | 0x00：0(dB) 0x01：3(dB) 0x02：6(dB) 0x03：9(dB) 0x04：12(dB) 0x05：15(dB) 0x06：16(dB) |
| IF_G | Byte | out | 0x00：12(dB) 0x01：18(dB) 0x02：21(dB) 0x03：24(dB) 0x04：27(dB) 0x05：30(dB) 0x06：36(dB) 0x07：40(dB) |
| Thrd | DWORD | out | When the threshhold is lower, the minimum RSSI that could be demodulated is lower, yet more unstable. Demodulation will totally fail if RSSI is too low. On the contrary, higher threshhold means only higher RSSI could be demodulated and more stable. 432 is the lowest suggested value. |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

Scan Jammer on RF Input

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD ScanJammer(LPBYTE CH_L, LPBYTE CH_H, LPBYTE JMR); |  |  |  |
| Remarks |  |  |  |
| Scan jammer on RF input to check jammer strength on each channel. |  |  |  |
| Parameters | DataType |  | Description |
| CH_L | Byte | out | Index of first channel |
| CH_H | Byte | out | Index of last channel |
| JMR | Byte[ ] | out | Each byte refers a strength value of a channel；For example：(0xF2 is for 14dBm) Conversion formula： int jammer = JMR[n]; if (jammer > 127) { jammer = -((-jammer) & 0xFF); } |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

Scan RSSI

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD ScanRSSI(LPBYTE CH_L, LPBYTE CH_H, LPBYTE JMR); |  |  |  |
| Remarks |  |  |  |
| Scan RSSI on RF input to check if any reader is currently working. |  |  |  |
| Parameters | DataType |  | Description |
| CH_L | Byte | out | Index of first channel |
| CH_H | Byte | out | Index of last channel |
| JMR | Byte[ ] | out | Each byte refers a strength value of a channel；For example ：(0xBA 为-70dBm) Conversion formula： int RSSI = JMR[n]; if (RSSI > 127) { RSSI = -((-RSSI) & 0xFF); } |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

NXP ReadProtect/Reset ReadProtect

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD NxpReadProtect(LPBYTE AccessPassword, BYTE Protect, LPBYTE PC, LPBYTE EPC); |  |  |  |
| Remarks |  |  |  |
| NXP G2X RFID supports ReadProtect/Reset ReadProtect command. When ReadProtect command is executed, the ProtectEPC and ProtectTID of the RFID will be set to 1, and it will get into the state of data protecting. In order to cancel data protecting and come back to normal, Reset ReadProtect should be executed. Select command must be sent before this operation to specify single RFID. |  |  |  |
| Parameters | DataType |  | Description |
| AccessPassword | Byte[ ] | in | Access Password |
| Protect | Byte | in | 0x00：ReadProtect 0x01：Reset ReadProtect |
| PC | Byte[ ] | out | PC of the RFID to be operated |
| EPC | Byte[ ] | out | EPC of the RFID to be operated |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

NXP Change EAS

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD NxpChangeEas(LPBYTE AccessPassword, BYTE Protect, LPBYTE PC, LPBYTE EPC); |  |  |  |
| Remarks |  |  |  |
| NXP G2X RFID supports Change EAS command. When Change EAS command is executed, the PSF of the RFID will be set to 1 or 0. when PSF of the RFID is set to 1, RFID will respond to EAS_Alarm command, otherwise it won’t. Select command must be sent before this operation to specify single RFID. |  |  |  |
| Parameters | DataType |  | Description |
| AccessPassword | Byte[ ] | in | Access Password |
| Protect | Byte | in | 0x00：Set PSF to ’0’ 0x01：Set PSF to ’1’ |
| PC | Byte[ ] | out | PC of the RFID to be operated |
| EPC | Byte[ ] | out | EPC of the RFID to be operated |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

NXP EAS_Alarm

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD NxpEasAlarm(LPBYTE EASAlarmCode); |  |  |  |
| Remarks |  |  |  |
| NXP G2X RFID supports EAS_Alarm command. When a RFID receives EAS_Alarm command，it responds 64bits EAS-Alarm code immediately. Notice that only when PSF of the RFID is set to 1 will it respond to this command. This command contributes to a shoplift-preventing system. |  |  |  |
| Parameters | DataType |  | Description |
| EASAlarmCode | Byte[ ] | out | 64bits EAS-Alarm code |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

NXP ChangeConfig

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD NxpChangeConfig(LPBYTE AccessPassword, LPBYTE Config, LPBYTE PC, LPBYTE EPC); |  |  |  |
| Remarks |  |  |  |
| Some series of NXP G2X RFIDs (such as G2iM and G2iM+) support ChangeConfig command. 16bits Config-Word of the NXP G2X can be read or modified by this command. Config-Word of a NXP G2X RFID is located on Memory Bank 01（EPC zone）at offset 20h (word address, which can be read by common Read command. When RFID is at the state of Secured (Secured State), Config-Word can be modified. Notice that modifying Config-Word means flipping data bits, to be specific, flip bits (1 to 0, 0 to 1) where you input an 1 and hold where you input a 0. Select command must be sent before this operation to specify single RFID. |  |  |  |
| Parameters | DataType |  | Description |
| AccessPassword | Byte[ ] | in | Access Password |
| Config | Byte[ ] | in,out | 0x0000(returns original Config-Word，same as reading) |
| PC | Byte[ ] | out | PC of the RFID to be operated |
| EPC | Byte[ ] | out | EPC of the RFID to be operated |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

Impinj Monza QT

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD ImpinjMonzaQT(LPBYTE AccessPassword, BYTE RW, BYTE Persistence, BYTE Payload, LPBYTE PC, LPBYTE EPC, LPDWORD QTControl); |  |  |  |
| Remarks |  |  |  |
| Impinj Monza 4QT RFID supports QT command, which modifies the QT Control word of a RFID. Setting QT_SR can cut down operating distance when a RFID is at Open state or at Secured state or about to change state into Open or Secured. Modifying QT_MEM can switch the RFID between using Public Memory Map and using Private Memory Map. Select command must be sent before this operation to specify single RFID. |  |  |  |
| Parameters | DataType |  | Description |
| AccessPassword | Byte[ ] | in | Access Password |
| RW | Byte | in | 0x00: Read 0x01: Write |
| Persistence | Byte | in | 0x00: Write into volatile storage area 0x01: Write into non-volatile storage area |
| Payload | Byte | in | 0x01: QT_SR 0x02: QT_MEM |
| PC | Byte[ ] | out | PC of the RFID to be operated |
| EPC | Byte[ ] | out | EPC of the RFID to be operated |
| QTControl | Byte[ ] | out | QT Control Word |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

NetCfg_Open

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD NetCfg_Open(LPSTR IP); |  |  |  |
| Remarks |  |  |  |
| Access to NetCfg configurations function. |  |  |  |
| Parameter | DataType |  | Description |
| IP | String | in | IP address to bind |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

NetCfg_Close

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD NetCfg_Close(); |  |  |  |
| Remarks |  |  |  |
| Close NetCfg configurations. |  |  |  |
| Parameter | DataType |  | Description |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

NetCfg_SearchForDevices

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD NetCfg_SearchForDevices(BYTE DeviceType, LPDWORD Count, LPBYTE Data, LPDWORD Length); |  |  |  |
| Remarks |  |  |  |
| Search for NetCfg devices. |  |  |  |
| Parameter | DataType |  | Description |
| DeviceType | BYTE | in | Type of the device |
| Count | DWORD | out | Number of devices which responded. |
| Data | NET_DeviceInfo[ Count] | out | NET_DeviceInfo structure； typedef struct NET_DeviceInfo { BYTE MAC[6]; //MAC Address BYTE IP[4]; //IP Address BYTE VER; //Version BYTE LEN; //Length of the name of devices BYTE NAME[16]; //name of devices }; |
| Length | DWORD | out | Length of data |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

NET_DEVICE_CONFIG Structure

typedef struct NET_DEVICE_CONFIG

{

DEVICEHW_CONFIG     HWCfg;			 /*Configuration info of devices*/

DEVICEPORT_CONFIG   PortCfg[2];          /*Configuration info of children devices*/

}

DEVICEHW_CONFIG Structure

typedef struct DEVICEHW_CONFIG

{

UCHAR  bDevType;				    /* Type of the device (please refer to Table of type of devices) */

UCHAR  bAuxDevType;				/* Aux type of the device */

UCHAR  bIndex;					/* Index of the device */

UCHAR  bDevHardwareVer;			/* Hardware version of the device */

UCHAR  bDevSoftwareVer;			/* Software version of the device */

UCHAR  szModulename[21];		    	/* name of the module */

UCHAR  bDevMAC[6];				/* MAC address of the module */

UCHAR  bDevIP[4];    				/* IP address of the module */

UCHAR  bDevGWIP[4];    			/* Gateway IP address of the module */

UCHAR  bDevIPMask[4];    			/* IP Mask of the module */

UCHAR  bDhcpEnable;				/* DHCP Enable. DHCP Status,1:Enabled，0：Disabled*/

UCHAR  breserved[29];				/* Reserved property*/

};

DEVICEPORT_CONFIG Structure

typedef struct DEVICEPORT_CONFIG

{

UCHAR  bIndex;			/* Index of the port */

UCHAR  bPortEn;			/* Is port Enabled: 1：Enabled ；0：Disabled */

UCHAR  bNetMode;		/* Net mode: 0: TCP SERVER;1: TCP CLIENT; 2: UDP SERVER 3：UDP CLIENT; */

UCHAR  bRandSportFlag;	/* Is local port random (TCP CLIENT)，1：random 0: not random*/

USHORT wNetPort;			/* Port */

UCHAR  bDesIP[4];    		/* destination IP address */

USHORT 	wDesPort;			/* Local port (as TCP SERVER) */

ULONG  dBaudRate;		/* Baud rate of serial port: 300---921600bps */

UCHAR  bDataSize;			/* Bits which indicate size of data on serial port: bit5–bit8 */

UCHAR  bStopBits;			/* Stop bits on serial port: 1: one stop bit; 2: 2 stop bits */

UCHAR  bParity;			/* Check bits: 0: Odd parity; 1: Even parity; 2: MARK bit is 1; 3 SPACE(set bits to 0); */

UCHAR  bPHYChangeHandle;	/* what should be done to socket when PHY disconnects，1：close socket 2、nothing*/

ULONG  dRxPktlength;		/* Package size of Rx data on serial port. (1024 at max) */

ULONG  dRxPktTimeout;		/* Timeout of Rx data on serial port (/10ms), 0 means infinite timeout */

UCHAR  bReConnectCnt;	/* Max times of trying to connect to TCP SERVER (when working as TCP CLIENT)*/

UCHAR  bResetCtrl;			/* Reset control on serial port: 0: preserve data buffer; 1: flush data buffer on serial port when connected */

UCHAR  bDNSFlag;			/* Is DNS enabled，1：Enabled. 2：Disabled*/

UCHAR  szDomainname[20];	/* domain name*/

UCHAR  bDNSHostIP[4];		/* DNS host*/

USHORT wDNSHostPort;		/* DNS port*/

UCHAR  breserved[8];		/* Reserved property */

};

NetCfg_GetInfo

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD NetCfg_GetInfo(BYTE DeviceType, LPBYTE MAC, LPBYTE Data, LPDWORD Length); |  |  |  |
| Remarks |  |  |  |
| Get NetCfg of device |  |  |  |
| Parameter | DataType |  | Description |
| DeviceType | BYTE | in | Type of device |
| MAC | Byte[ ] | in | MAC address |
| Data | NET_DEVICE_CONFIG | out | Data Structure |
| Length | DWORD | out | length of data |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

NetCfg_SetInfo

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD NetCfg_SetInfo(BYTE DeviceType, LPBYTE LocalMAC, LPBYTE DevMAC, LPBYTE Data, DWORD Length); |  |  |  |
| Remarks |  |  |  |
| Set NetCfg of device |  |  |  |
| Parameter | DataType |  | Description |
| DeviceType | BYTE | in | Type of device |
| LocalMAC | Byte[ ] | in | Local MAC address |
| DevMAC | Byte[ ] | in | MAC address of the device |
| Data | NET_DEVICE_CONFIG | out | Data Structure |
| Length | DWORD | out | length of data |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |

NetCfg_FactoryReset

| Function |  |  |  |
| --- | --- | --- | --- |
| DWORD NetCfg_FactoryReset(BYTE DeviceType, LPBYTE MAC); |  |  |  |
| Remarks |  |  |  |
| Reset configuration |  |  |  |
| Parameter | DataType |  | Description |
| DeviceType | BYTE | in | Type of device |
| MAC | Byte[ ] | in | MAC address |
| Return |  |  |  |
| Return | DWORD | out | Return value: 0 for success |